Observatories Report To EVN TOG

INAF-Institute of Radioastronomy

Medicina station		Period November 2020 - April 2020

A. Orfei, G. Maccaferri, A. Orlati

1) Antenna

No news with respect to the last TOG report on November 2020.

2) Receivers

We are investigating the nasty ripples on X- and K-band receivers.

At present the frontend capable to provide at least 1 GHZ bandwidth is the K-band receiver. X-band and C-band receivers are 800MHz each.

The PON grant will allow to install a tri-band    (K,Q and W) capable of 8GHz bandwidth. The receiver will be available in 2022.

A new dual feed, KU-band (13.5-18 GHz) receiver is planned within next two/three years . The bandwidth will be 4GHz.

3) VLBI terminal

Our Flexbuff system is presently equipped with 360TB.

jive5ab 3.1.0, 64bit, released 28/09/2020

We're running DBBC V107

PFB with v16 firmware

Fila10G with V4.1 231118 firmware

We are on the verge to increment our disk pool by 500TB together with a new Flexbuff. We plan to sent disks to JIVE to reach 500TB, by the end of the year.

According present schedule a new DBBC3 will be delivered to Medicina by December 2021.

Medicina is connected to GARR backbone at 10Gbit/sec.

Mark5C was decommissioned.

4) Field System

We are running the new FS 10.0.0 on a FSL10 Debian machine

5) VLBI sessions

EVN 2020-3

Session fully completed. Fringes during all the NMEs, no evidence of data loss during user experiments.

EVN 2021-1

Session fully completed. Fringes during all the NMEs, no evidence of data loss during user experiments.