Noto Station Report

From Noto staff (INAF-Institute of Radioastronomy)

To EVN-TOG

21/04/2021

Reference period Novemberer 2020 - April 2020

1) Antenna.

In our previous report to the TOG    we outlined a failure on the mechanics of the Z Axis (focusing) of the servo system that moves the box of the primary focus

receiver into position. The problem was due to a seized recirculating sphere screw which blocks the actuator. This failure does not allow focus

switching so only primary focus frequencies are available (L-,S- and X-band), C-,M- and K-band are not.

Required spare parts were procured and the component repaired last week. We are now performing the required checks and the secondary focus frequencies

will probably be available for next session.

Two new UPSs was purchased at the end of the year. The units will be installed next weeks are will serve both the station

(control room and T&F lab included) and the telescope.

2) VLBI terminal

FS Version: 9.13.1-rc2

DBBC fw version: DDC V107, PFB V16

FiLa10G fw version: 4.1

Flexbuff sw version: 2.8.1

Flexbuff disk space: 360 TB

DDBC had some failures related to power distribution board, the power supply and the CoMo board. The backend was promptly repaired, also we received some spare parts from Bonn.

Anyway a major renovation is required. We are planning to ship the rack to Bonn. This will tentatively be done in late summer.

We are on the verge to increment our disk pool by 500TB together with a new Flexbuff. We plan to sent disks to JIVE to reach 500TB, by the end of the year.

According the present schedule a new DBBC3 will be delivered to Noto by December 2021.

Noto is connected to GARR backbone at 10Gbit/sec.

3) Receivers

At present no frontend is capable to provide at least 1 GHZ bandwidth.

The PON grant will allow to install a tri-band    (K,Q and W) capable of 8GHz bandwidth. The receiver will be available in 2022.

No other receivers are planned in the next two/three years.

4) VLBI sessions

Session 1-2021:

L-band NME failed because of local oscillator of the downconversion system was not properly locked. The problem was fixed before the beginning of the first user experiment.